# Sride1

Good morning lady and gentelman, Welcome and thank you for attending the General Motors Safety 21  Design – In Safety Process

Today I will review the Safety 21 Design In Safety Process that GM corporations and GM Thailand use this process as the tools for designing new automotive product and process.

Sride2

In the presentation I will review

Introduction to Design In Safety.  This  review will cover the history ,concepts, overview and highlights of the GM common design for health and safety specification-DHS

Sride3

Design – in safety is a strategy which promotes the integration of health and safety requirement in the product and process concept at the design stages

Sride4

Properly design – in safety will help to eliminate human exposure to all hazards.Also safeguards will not be used unless easy to use or easy to recover by the operators

Sride5

In this slide you will see why do we need design – in.at you  can see on the left hand side. Traditional concept less design involvement at the beginning  then will have  high effect to effort , cost and resources.

On the right hand side future concept more design involvement at the beginning then will have less effect to effort , cost and resources.

Sride6

So high influence of design in  will incur less cost in the long term , on the other hand low influence of design in will incur more cost in the long term

Sride7

Hierarchy of controls design preferences. At you can see number one elimination and substitutions is the least human involvement  then you go down to  number five personnel  protective equipment is the most human involvement. With proactive design in will be the most effective , reactive design in will be the least effective.

Sride8

Design for health and safety specification will cover machinery and equipment , safety 21 process and tools, task bask risk assessment process and guideline and specification.

Sride9

This guidelines and specifications will cover the 14 items as the following

1. Workstation design/ operator interface, then 2, 3, 4 to 14

Sride10 Nil

Sride11

1.Workstation design / operator interface in this slide,  mostly in this design will refer to GENERAL MOTORS CORPORATION MANUFACTURING ERGONOMICS RECOMMENDATIONS ERG 1.2

Design should accommodate a wide range of human in term of reach,clearance,movement and work repetition Design and position operator control,displays,labels,Machine component and tool where operator has clear ,unobstructed manual and visual access and to provide a clear view of the entire cell

Use control display that are meaningful and easy to understand

Adequate lighting shall be provide during de-energized period

Sride12

2 .Workstation design / operator interface tool design in this slide

Power tools shall be equipped with a constant pressure switch to ensure that power is shut off when pressure is released from the power switch

The manufacturer’s rated speed (RPM) shall be clearly marked on all pneumatic power tool

Power tools that shoot nails,rivets or staples shall be equipped with a special device to keep fasteners from being ejected unless the working end is pressed against the work surface

Sride13

Electrical power operated tools shall be grounded or be double insulated

The manufacture’s rated speed shall be clearly marked on portable grinders and grinding wheels.

The maximum RPM of the grinding wheel must be rated equal to or higher than the manufacture’s rated speed for the grinder

The power tool shall be equipped with the appropriate tool rest,that  you can on the picture

Sride14

3. Maintainability, this is most importance of both operators and maintenance team.

As you can see in the picture.

All primary energy-isolation devices should be located near cell entrance points to minimize the time required for lockout and system restart

Design for safe access, working space,and clear cell visibility within and around all control panel

Design and install direct task lighting as part of a system to provide greater visibility during routine maintenance and service

Sride15

In the area that motorized lift equipment is not available , you should design hoist rail into the system.

And also the manufacturer shall provide information on rigging and lift system.

The follow as Center of gravity/Total weight of the system/Designated lift points/Specifying the require working load limit

Sride16

Fall and Working Surface ,Whenever performance of any task would allow a worker to fall a distance where the likelihood of a serious or fatal injury exist the hazard of falling must be identified,evaluated and control

At you can see in the picture the shutoff valves,monitoring & maintenance equipment should be located at readily accessible floor or grade floor

Sride17

For climbing applications,are fixed stairs used in preference over alternating tread type stairs or vertical ladder

If fixed stairs can not be used due to limited space application are alternating tread-type stair will be used.

In the event neither of the above can be used,vertical ladder should be use only for infrequent accessed maintenance location

Sride18

All step, ladder rungs and working surface must have a“non-slip” surface

Consideration shall be given to elevating cable,pipe,hose and conduits should minimize tripping hazards,If elevation of cable,pipes,hoses is not possible,they shall be covered as you can see in the picture

Holes in floors or platforms shall be properly marked and guarded

Sride19

In this slide I will talk about Industrial Hygiene

All Material Safety Data Sheet (MSDS) from supplier must meet GM requirements

All chemical materials proposed for using in research and production shall be approve before purchase

All machinery,power tools been design noise level not to exceed 80dBA

All machinery been design to assure impulse sound pressure level not exceed 130 dBA

Sride20

Fire Prevention Plan

Fireproof cabinet shall be provide to store flammable substances and each cabinet shall be properly marked identifying it as a flammable storage cabinet as you can see in the picture

Equipment that contain a flammable substance shall be designed to prevent accidental ignition

Fire fighting equipment shall be stationed in the work area an shall be properly marked to indicate its use

Sride21

Safeguarding device:Guard

A guard needs to be constructed to prevent reaching over under,around or through to a point of hazard

A guard needs to be designed and install with an understanding of what tasks need to be per formed on the equipment it is design to protect

Guard shall be free of sharp edges and protections and shall not themselves create a hazard

Sride22

As you can see in the picture , A guard shall be constructed where required to allow for visibility into the hazard area to reduce the need for access beyond the guard

A guard shall contain parts and equipment within the guarded area from being ejected to outside of the guard area

Sride23

Safeguarding device:Interlocks

As you can see in the picture ,Safety Interlock system for gate and guard shall:

Be control reliable device and installed in a control reliable manner

Prevent the use of unauthorized and or unintentional bypass device

Be used where hinged or sliding door or guards provide access to hazard

Be appropriate for the task and related hazard

Sride24

Safeguarding device:

Presence Sensing Devices Presence sensing devices shall be control reliable and be installed in a control reliable manner

Be installed per the safe distance requirement under the OSHA

Indicator light shall provide to indicated the device is function.If used for operator protection the indicator lights shall be observable by the operator

Sride25

If there is a loss of power the PSD shall initiate an immediate stop command to the equipment it is controlling

Awareness devices should be added to prevent inadvertent trip of the PSD

Sride 26

Safeguarding device: Two Hand
 Control Device

As you can see in the pictures Be protected against unintentional operation

Be arrange by design,construction or separation to require the concurrent use of both hand within 500 milliseconds s to initiate cycle

Be designed to require the release of all operator’s hand control and the re-activation of all operator’s hand control before a cycle can be initiated

Whenever  operator remove hand must issue stop signal from the control during hazardous portion of the cycle

Sride 27

Emergency Stop Function and Device as you can see in the picture

Be provide at each operator station where hazardous  motion exists

Be hardwired into the emergency stop circuit, can not use soft circuit.

Function independently from the system programmable electronic system

Not be of the automatic resetting type,but shall require manual reset by operator

Not initiate,restart or create any hazardous motion when reset

Sride 28

Emergency stop will be red in color

Be palm type or mushroom-shaped

Have any existing background for the device actuator colored yellow

Be distinguishable from all other controls by size and color

Be unguarded and not recessed

Sride 29

Awareness Barriers Device

Awareness barriers example guardrails and chains on post should be provided a point of physical contact before entering the hazardous area and warn personnel of a present or approaching hazard

Awareness device shall be designed constructed and located to provide a recognized signal (audible,visual or a combination)

Sride 30

Lockout Energy Control

The control circuit and primary energy isolating device shall be design to provide easy and safe system access for service and maintenance task

Lockout/energy control placard that identify the store energy sources and the safeguard against their release shall be provided this will help maintenance team  easy to locate the primary isolating devices.

When dropping or drifting of equipment /part present hazard ,device shall be install  to eliminate hazard

Master Iockable electric disconnects and shut-off valve (Pneumatic and Hydraulic) shall be install on the service side,outside of the cell , this will prevent product line stop or interrupt the system.

Sride 31

Monitored Power Systems Application

All of electrical wirings should be designed with these requirements

Fail to Safe

Safety systems stop in a safe condition

Independent

Allow a comprehensive test sequence to be written

Detects if the safety circuitry has been bypassed or removed

Redundancy

Paralled or Series Devices

Monitoring

Internal and/or External Feedback

Sride 32

In this slide will talk about Safety Design Review (SDR)

A SDR consists of the GM DHS 3 Design In Specification in question format.

A SDR is conducted at the pre-quote, final design, final build stage and then reviewed prior to turn over of the equipment to production team.

To help you understand more SDR consists of 3 steps

SDR 1 will be used during pre-quote ensures all applicable sections are included in the quote

Sride 33

SDR 2 will be used during final design to  ensure that our specifications are part of the equipment design, safety controls, etc.

SDR 3 will be used during final build to  ensure that all of the specifications we asked for are included and functioning.  Can also be no argument on who is responsible if the specification requirement is not included or functioning.

Sride 34

SDR Benefits to

On operators and Skilled Trades will

Reduces potential for personal injury

Also reduces the amount of human involvement to work safely

And improves employee moral

On engineers and Purchasing side will

Reduces re-engineering when not necessary

Also reduces complaints from end-users

And reduces potential modification costs later (retrofit)

On  Company will Increases compliance with all known safety regulations

Also reduces initial equipment build costs

Reduces initial program start-up costs

Supports common global initiatives

Sride 35

What Is Task Based Risk Assessment (TaBRA)?

It is a systematic approach to identify all tasks and the hazards associated with each task.

It provides a methodology to select the appropriate safeguard based on each task/hazard combination.

It provides a methodology to verify that the safeguard has reduced the risk of injury during the performance of the task.

Sride 36 Nil

Sride 37

At you can see in this slide all plants input the TaBRA.  data into master base , then designers will utilize the TaBRA data to put all associated equipment in the new cell to eliminate all hazards then get reviewed and sign off  by safety and all concern persons on the lay out.

All parties input recommendations to eliminate/reduce risk and verify complied TaBRA.

Sride 38

TaBRA Benefits to:

On operators and Skilled Trades will have

“Hands On” involvement in equipment safety process

Seeks their input, suggestions and solutions

Improves employee moral

On engineers and Purchasing will

Ensures its done right the first time

Eliminates re-engineering of safety equipment

Reduces complaints from end-users

Reduces potential modification costs later (retrofit)

On company

Improves initial equipment build schedule

Reduces initial program start-up costs

Supports common global initiatives

Results captured to eliminate “reinventing the wheel”

Sride 39

So Safety 21 Process tools provides:as the influence stage by  using SDR, Continuity stage by using TaBRA and Confidence stage by using validation process.

Sride 40

On this slide you will see Manufacturing’ s machinery and equipment  life cycle of break into three phases , first design phase  will start from preliminary design  that include safety design analysis by using common template, SDR 1 ,  pre award (quote )and award PO

Pipeline phases  will start from reviewing detail design ,by using  SDR 2 & new equipment TaBRA,then  supplier build equipment or buy off the shelf and go to  SDR 3 Machinery & equipment acceptance.

Plant floor phase will start from M&E  installation , validation and run production